Shellfish Hatchery Water Sampling Guide

Jasmine Rusnell

Major Project/ Report submitted to the faculty of the Virginia Polytechnic Institute and State University in partial fulfillment of the requirements for the degree of

Online Master of Agricultural and Life Sciences

In

Food Safety and Biosecurity

David Kuhn -Department of Food Science and Technology

Renee Boyer

Kerry Redican

Date of Submission-5/5/2017

Keywords: Shellfish Aquaculture, water sampling, water protocols

Shellfish Hatchery Water Sampling Guide

Jasmine Rusnell

ABSTRACT

The purpose of this peer reviewed extension article was to create a guide that helps hatcheries with proper water sample storage and protocols that are easy to read and understand. Shellfish aquaculture has become a profitable industry and will continue to grow however, water quality issues can drastically affect this. These issues have been causing problems in the Chesapeake Bay where oysters were already experiencing a decrease in population. To determine the cause of water quality issues it’s essential to take samples throughout the hatchery season therefore following proper sampling and storage protocols is a top priority for hatchery personnel. Water sample testing might include analysis of Harmful Algal Blooms, pesticides, herbicides, metals, and bacteria. Testing for these require sending samples to analytical laboratories for analysis.

For this project I built upon the research and work already being conducted in the Chesapeake Bay by faculty and students at Virginia Tech. I searched for more supporting documents and collected different pieces of information from multiple faculty members to put it all together into an extension article. Additionally I created some tables to assist hatchery personnel. For example upon determining the need for water quality testing, hatchery personnel can exam the list of analytical labs and contact information as shown in the document to decide where to send their samples to. With proper water sampling protocols shellfish hatcheries can detect water quality issues sooner and determine the best actions to deal with their problem.

Table of Contents

Introduction ……………………………………………………………………………………...4

Review of Literature …………………………………………………………………………….5

Project Methodology……………………………………………………………………………..6

Project Results, Discussion, and Recommendations …………………………………………....13

References ……………………………………………………………………………………….13

Appendices……………………………………………………………………………………….14

Introduction

- Background and Setting

Oysters are known for filtering the Chesapeake Bay however, there have been drastic declines in oyster populations due to overfishing and disease (Bricker, Bricker III, and Rice 2014). Shellfish hatcheries are important to increase the number of oysters in the bay via planting new populations, as well as, helping to offset the high consumer demand. This realization is allowing shellfish hatcheries to become a profitable aquaculture industry. In Virginia, oyster and clam hatcheries observed a noticeable decrease of larvae production during multiple hatchery seasons. The high larval mortality rate during some of these low production periods could be attributed to multiple factors.

The Chesapeake Bay has a lot of pollutants that are potentially harmful to shellfish larvae. It was determined that the best way to decide what is causing the detrimental effects on the larvae is for the hatcheries to take samples for water quality testing. Samples should at the very minimum be collected whenever a production failure occurs if not throughout the hatchery season. Hatcheries will need to monitor pH, ammonia, nitrate, nitrite, salinity, carbon dioxide, temperature, and calcium levels. Necessary testing of the water samples includes harmful algal blooms, metals, bacteria levels, herbicides, and pesticides. Some of the water testing has to be conducted in a commercial lab e.g. testing for metal toxicity or algal blooms. Collection and storage of these samples is different for each of the water quality areas.

For the Harmful Algal Bloom water sample collection process, two identical samples in coliform water bottles have to be collected. These samples should be wrapped in a wet towel to keep them cool. One of the samples is used for microscopic identification and upon determination of any algal species present the other sample would be sent to a lab for testing.

Metals unlike harmful algal bloom samples, are not filtered and are collected in clear plastic containers. Care must be taken to ensure that no contamination takes place throughout the collection process. Upon completion of the sampling collection process, the samples that will not be analyzed immediately have to be acidified. If acidification is not possible then it should be noted that the samples are unacidified.

Similar to metals, the bacteria collection process requires ensuring that no contamination occurs. These samples can be collected in glass or plastic containers. Identical samples are necessary for determining the precision of the laboratory. When samples are sent to the lab a sample of sterilized water in a sterilized bottle should be sent with it. This will also help ensure that no contamination occurred during the collection process.

Herbicide sampling like bacteria and metals is susceptible to contamination. Samples either need to reach a laboratory for analysis within 24 hours or else they have to be acidified. Three water samples should be collected in Teflon-lined jars. They should be stored on ice and in the dark for preservation.

Pesticides like herbicides should be stored in the dark and refrigerated for preservation. To assist with preservation 1000 mg of ascorbic acid should be added to 1 liter of the sample suspected of having residual chlorine. Properly cleaned glass containers are needed for collection.

Protocols for water quality testing are necessary for each hatchery to check for any presence of biological agents in the water. The data collected from water quality testing will allow the identification of a potential water quality issue that could be causing more larval production problems. Upon identification of the problem, solutions can be offered and implemented.

- Statement of the Problem

Water quality issues are affecting the production of shellfish larvae development during a hatchery season.

- Purpose of the Project

To create a guide that helps hatcheries with proper water sample storage and sampling protocols that is easy to read and understand.

- Definition of Terms

Denitrification: the reduction of nitrates and nitrites generally by bacteria that can result in the release of nitrogen into the air.

Electron-Capture Detectors: a device that detects molecules in a gas via the attachment of electrons.

Gas Chromatography: a process where the mixture is vaporized and carried by a stream of gas which separates components.

Harmful Algal Blooms (HAB): the rapid increase of algae in freshwater or marine environments that can be toxic.

- Significance of the Problem

The importance of proper sampling cannot be emphasized enough. Proper water sampling is essential to acquire accurate and meaningful results. Without the right water sampling techniques it would be hard to determine the cause of larval production problems.

Review of Literature

- Review of Literature

Habitat destruction, over harvesting, and diseases have been destroying the oyster population in the Chesapeake Bay. Due to this many scientists suggested halting the harvest of wild oysters and promoting oyster aquaculture. The social acceptance of shellfish aquaculture is currently the restricting factor for expanding the industry (Rose, Bricker, Tedesco, and Wikfors 2014).

- Theoretical or Conceptual Framework

The scientists in favor of oyster aquaculture believe that it can remove nutrients such as carbon, nitrogen, and phosphorous from the environment (Ray, Li, Kangas, and Terlizzi 2015). Excess nutrients can cause multiple problems including hypoxia, harmful algal blooms, and loss of habitats (Rose, Bricker, Tedesco, and Wikfors 2014). Generally the nutrient known as the limiting factor is Nitrogen. Nitrogen can come from inorganic and organic matter. It can be assimilated by phytoplankton which is then consumed by shellfish. The nitrogen from the phytoplankton is then integrated into the oysters’ shells and tissues. When oysters are harvested from the estuary or farm the nitrogen is taken out of the local environment. An additional benefit some scientist discovered was having submerged gear and oyster cages on the ocean floor. Their research concluded that there were more fish around the submerged gear than other areas with muddy bottoms (Williamson, Tilley, and Campbell 2015).

Others disagree that there is a benefit to oyster aquaculture and a few of these disputes have led to lawsuits. Their argument was that the aquaculture industry operations would disturb commercial and recreational activities. The main argument is that oyster aquaculture systems compete for space with the submerged gear; space that is essential for the habitat of fish (Vaudrey, Getches, Shaw, Markow, Britton). A common example is eel grass although, the data collected by Vaudrey concluded that the aquaculture gear had no effect on the growth of eel grass (Vaudrey, Getches, Shaw, Markow, Britton). Other research on how shellfish aquaculture systems affect the environment was inconclusive. Another debate by many scientists is the possibility of unintentionally introducing a non-native oyster species into the Chesapeake Bay (Methratta, Menzie, Wickwire, and Richkus 2013). Additionally there is concern about how many oysters it would take to restore the whole estuary. This is because the natural denitrification process varies in space and in time. Current quantification of the denitrication process in oyster habitats that have been restored has proven to be difficult (Rose, Bricker, Tedesco, and Wikfors 2014). Further testing and study is needed to increase the accuracy of this quantification process.

- Summary

Although debates have been going back and forth for a decade regarding shellfish aquaculture the overall conclusion is that it will help the Chesapeake Bay. All of the research conducted spanned over a few years. Most of the studies resulted in favor of oyster aquaculture. The few that were not in favor of aquaculture did not generate conclusive results. Additional studies should be done to collect more data and achieve accurate results so informed decisions can be made.

Project Methodology

- Targeted Population and Participating Audience

The targeted population is shellfish hatchery growers as well as juniors and seniors in high school. The information presented in the extension article is meant to be a water sampling guide for the above mentioned target population.

- Extension Article

Virginia Cooperative Extension

Shellfish Hatchery Water Sampling Guide

Jasmine Rusnell, Masters Student, Virginia Tech; David Kuhn, Assistant Professor and Extension Specialist, Department of Food Science and Technology, Virginia Tech; Karen Hudson, Commercial Shellfish Aquaculture Specialist, Virginia Institute of Marine Science; Aaron Beck, Assistant Professor, Virginia Institute of Marine Science; Michael H. Schwarz, Extension Specialist Director, Virginia Seafood AREC; Kimberly Reece, Professor of Marine Science, Virginia Institute of Marine Science

This guide is meant to inform shellfish hatcheries of proper sampling and storage of water samples for analysis of harmful algal blooms (HABs), metals, pesticides, herbicides, and bacteria. The collection and preservation of these samples is summarized in Table 1.

Shellfish hatcheries are the base of an emerging and profitable aquaculture industry. During a hatchery season, it is common for unknown water quality issues to impair production for short periods of time and in some extreme cases, for a significant portion of the production season.  One useful tool is to collect discrete samples during production failure and/or to archive water samples throughout the hatchery season for later analysis if a production problem is experienced after the fact. Proper protocols for sampling and, storage prior to analysis are required to obtain accurate and meaningful results.

Harmful Algal Bloom (HAB) Sampling and Preservation

The first step in the Harmful Algal Bloom (HAB) water collection protocol is to get two water samples (replicate samples) from each sampling location. One sample will be used for microscopic identification and if it contains any algal species of interest, or if it was from an algal bloom, then the other sample will be filtered for genetic analysis.

Replicated samples need to be collected in 120 mL disposable plastic coliform water bottles. Coliform water bottles are sterile until opened and generally are sealed. These bottles should at least be filled to the 100 mL mark. Afterwards the samples should be wrapped in wet paper towels to keep them cool.

One of these collected samples will be used for microscopic identification and the determination of the number of cell types. After the sample has been wrapped in a wet paper towel it should be delivered promptly to the testing facility. This is considered a “live” sample. If the sample is not available within 18-24 hours for analysis then 400 μl of Lugol’s solution should be added to the sample for future evaluation. Lugol’s solution contains 10 g iodine crystals, 20 g potassium iodide, 200 mL distilled water, and 20 mL glacial acetic acid. Once mixed, this needs to be stored at room temperature.

The second sample collected for HAB sampling should be filtered as soon as possible to reduce the possibility of a change in the composition of the sample. The filtration process begins with attaching a funnel adapter and stopper to a test filter funnel containing a 3m Isopore™ membrane filter (Millipore Corp, Darmstadt, Germany). The funnel adapter should then be placed into the neck of the flask with a filter funnel placed on the rim. Filter a total of 100 mLs of the sample unless that is not possible due to turbidity. When a smaller volume is filtered, the volume needs to be recorded. Using a disposable forcep, the filter should be removed, folded in half, and placed in a new Ziploc bag. The bag should be labeled and stored in a freezer at -20°C. The forceps, test filter funnel, and bottle can be disposed of after completing the filtration process. Bottles for water samples and Ziploc bags for filter should be labeled with location, date and time of collection. Temperature and salinity should be noted, if these data are available.

Metal Sampling and Preservation

Samples that are to be analyzed for total metal contaminants are not filtered. The following procedure is appropriate for non-volatile metals like copper, zinc and lead. The most basic approach is to collect an unfiltered water sample for “total” metals, which includes both phases and reduces the amount of required handling. Samples should be approximately 250 to 1000 mL but smaller volumes (10-100mL) are acceptable if levels are sufficiently high. Clear or translucent plastic containers usually have the lowest levels of common contaminant metals. Low density polyethylene, high density polyethylene, and polypropylene are all acceptable plastics.

Soaking the container in deionized water and 10% hydrochloric acid, known as an acid bath, is needed to clean the container. It should soak for a minimum of one week and then be rinsed with deionized water. These bottles can also be left soaking indefinitely until needed. After rinsing, bottles can be capped and stored in clean Ziploc-type plastic bags to avoid exterior contamination by dust. A convenient acid bath can be made from a new, clean 5 gallon bucket (preferably a white-colored bucket, as some pigments contain metal contaminants), and bottles can be left soaking indefinitely until use. (**Note: use of acids other than hydrochloric acid can deteriorate plastic bottles, making them brittle and unfit for sample collection.)

If acid washing is not available, new bottles should be used.

Before sample collection, both new and acid washed bottles must be rinsed 5 times with the water being sampled. Persons performing sampling should be careful to include the cap and bottle threads during rinsing, and to ensure that nothing touches the water sample or bottle interior except for the sample. Care must also be taken to avoid contamination from hair, dust, and other particles. Bottles should not be completely filled. There should be approximately 5% headspace to allow for acid addition and mixing.

Once the sample has been collected, the bottles need to be placed in Ziploc bags. To determine the amount of contamination from the bottle container, at least one should be put aside for testing. Samples that are ready to be analyzed right away (a few days) do not need to be acidified. For those samples that cannot be analyzed within a few days, acidification is necessary. Samples for metals analysis should be acidified after collection to pH <2 (usually ~2 mL concentrated HCl per liter sample) with high-purity metal-free acid (TraceMetal or Optima grade). If this is not possible, it should be documented that samples are unacidified. Acids can be added at a later time, but samples should be acidified for at least a week before analysis. Documentation should include the date, time, acidified/unacidified, filtered/unfiltered, and salinity measurement.

If samples are acidified and capped tightly, they can be stored (in bags) at room temperature for extended periods. The maximum recommended storing time is approximately 6 months. Generally commercial analytical labs will analyze these samples by mass spectrometry to determine the metal contaminates.

Pesticide Sampling and Preservation

The maximum holding time for testing for pesticides is a lot less than those for metals. The storage time for pesticides is 7 days. The minimum sample size used should be 1000 mL. A glass container rinsed with organic solvents such as Benzene, needs to be used when collecting a sample. Another option is for it to be rinsed in diluted hydrochloric acid and distilled water but it then has to be heat treated over night at 300° C (Goerlitz and Brown 1972). If the sample can’t be analyzed right away the sample should be kept out of light and refrigerated. To preserve a sample being tested for pesticides, 1000 mg of ascorbic acid should be added to 1 liter of the sample if residual chlorine is present (Eaton et al.1998). Whether or not the sample can be used right away, or if it has to be preserved, it should always be refrigerated.

Herbicide Sampling and Preservation

Herbicides also need to be refrigerated (4°C), no matter what, within 4 to 5 hours after collection (Goerlitz and Brown 1972).  Samples should be acidified with concentrated sulfuric acid at a rate of 2ml/liter. If the sample is not acidified then it has to reach the laboratory within 24 hours. Herbicides, like pesticides, can be analyzed by a few different detectors. Some of the detectors are more sensitive than others, which is why it’s imperative to use proper handling to prevent contamination. For example electron-capture detectors, which can be used in gas chromatographic analysis are very sensitive to interfering substances.

Approximately three 1000 mL samples need to be collected in a Teflon lined glass jar that was pre-rinsed before using. When collecting the sample there should be minimum air space in the top of the jar. The sample can be preserved immediately by being put on ice and placed in the dark (TCEQ 2012).

Bacteria Sampling and Preservation

Bacteria is a lot harder to test for due to the many different types of bacteria, natural levels of bacteria in the water, rainfall, and contamination. Bacteria is strongly affected by the amount of rainfall. Therefore depending on the season the data collected could result in an inaccurate outcome (EPA 2012). Additionally the analytical testing used for testing bacteria is not very accurate which is why preventing contamination is once again important.

Sample containers can range from glassware to plastic. If glassware or plastic is used or reused then they have to be rinsed and sterilized at 121°C for 15 minutes using an autoclave. Preferably, sampling is done with a presterilized disposable Whirl-pak bag because no additional preparation is necessary (EPA 2012). Sterilized bottles are also needed. Sterile water should be collected in the sterilized containers to be sent with the selected samples. This ensures that there are no errors or contamination in the collection/ analysis process. Duplicate samples should also be collected. This is where another sample is collected at the same place at the same time as the first sample. By collecting a duplicate sample a determination of the precision of the laboratory can be concluded. Once the samples are collected they should be brought to a lab for testing immediately because the maximum holding period is only 6 hours. These samples should be stored at 4°C. When transporting them, they have to be placed on ice (EPA 2012).

Some tests are easy to complete and analyze in the hatchery. However, others need to be done in a commercial lab or university. For example HAB samples should be sent to Virginia Institute of Marine Science (VIMS) for analysis and water samples consisting of bacteria, pesticides or herbicides need to be tested in a commercial analytical laboratory. Table 2 contains a list of some of these laboratories, along with their contact information. Each laboratory has different analytical capabilities, which can be found on their website or by calling their office.

| Table 1. Collection and Preservation of Samples |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Determination | Container | Minimum Sample Size (mL) | Sample Type | Preservation | Maximum Storage Recommended | Regulatory |
| HAB | P(C) | 100 | g,c | - Live sample: Do not filter. Store at room temperature - Filter and store at -20°C | 1. 18-24 hours 2. Deliver as soon as possible | 18-24 hrs |
| Metal | P(A), G(A) | 1000 | g,c | For dissolved metals, filter immediately, add HNO3 to pH˂2 | 6 months | 6 months |
| Pesticides | G(S), PTFE-lined cap | 1000 | g,c | Refrigerate, add 1000 mg ascorbic acid/L if residual chlorine present | 7 d | 7 d until extraction; 40 d after extraction |
| Herbicides | G(S) PTFE- lined cap | 1000 | g,c | Refrigerate | 7 d | 7 d until extraction; 40 d after extraction |
| Containers: P(C) =plastic coliform; G(A) or P(A)=rinsed with 1 +1 HNO3; G(S)=glass rinsed with organic solvents or baked. Sample Types: g=grab; c=composite Preservation: Refrigerate=storage at 4°C ± 2°C Regulatory: additional storage recommendations if any |  |  |  |  |  |  |

| Table 2. List of Commercial Analytical Laboratories1 |  |  |  |
| --- | --- | --- | --- |
| State | Laboratory Name | Contact Info | Website |
| VA |  |  |  |
|  | A & L Eastern Laboratories, Inc | (804) 743-9401 | http://al-labs-eastern.com/ |
|  | Air Water & Soil Laboratories, Inc. | (804) 358-8295 | http://www.awslabs.com/ |
|  | Analytics Corporation | (804) 365-3000 | http://analyticscorp.com/ |
|  | B & B Consultants, Inc. | (434) 372-3393 | http://bandbcons.com/ |
|  | Babcock & Wilcox Lynchburg Technology Center | (434) 522-5130 | http://www.babcock.com/Pages/default.aspx |
|  | E M S, Inc. | 276-228-6464 |  |
|  | EnviroCompliance Laboratories, Inc. - Ashland | (804) 550-3971 | http://www.envirocompliance.com/ |
|  | Environmental Hazards Services, LLC | (804) 275-4788 | http://www.leadlab.com/ |
|  | Environmental Monitoring, Inc. | (276) 679-6544 | http://www.emilab.com/ |
|  | Environmental Systems Service, LTD.- Culpeper | (540) 825-6660 | http://www.ess-services.com/ |
|  | Greenway Engineering, Inc. | (540) 662-4185 | http://greenwayeng.com/ |
|  | Hampton Roads Sanitation District Central Environmental Laboratory/ Wet Laboratory | (757) 460-4217 | http://www.hrsd.state.va.us/ |
|  | Inboden Environmental Services, Inc. | (540) 477-3300 | http://www.4ies.com/ |
|  | James R. Reed & Associates | (757) 873-4703 | http://jrreed.com/ |
|  | Jennings Laboratories Inc. | (757) 425-1498 | http://www.jenningslab.com/ |
|  | Metropolitan Solutions | (757) 853-4000 | http://metrosolutionsusa.com/ |
|  | Newport News Shipbuilding- A Division of Huntington Ingalls Industries | (757) 380-3244 | http://www.huntingtoningalls.com/ |
|  | Schneider Laboratories Global, Inc. | (804) 353-6778 | https://www.slabinc.com/content/ |
|  | Universal Laboratories - Hampton | (757)865-0880 | http://www.universallaboratories.net/ |
| 1 Partial list compiled from VA Division of Consolidated Laboratory Services VELAP-certifies laboratories list (01-29-2015) 2 Approximate; contact individual laboratories to confirm analytical capabilities for a particular sample type |  |  |  |

Project Results, Discussion, and Recommendations

- Project results

A new useful tool for shellfish hatcheries was created that will help them achieve quality results used to manage their production.

- Conclusions and Implications

The number of hatcheries in Virginia have been continuously growing over the past few years but have been experiencing larval production problems during the hatchery season. Through proper water sampling protocols accurate results can be generated. These results can help hatcheries identify the cause of production failures. If appropriate sampling is not conducted not only would the conclusion be wrong but then the problem will not be fixed. This could lead to further larval production failure.

- Recommendations

Regular monitoring of the water by conducting on-site testing will help with the awareness of water quality. It is also highly recommended for hatchery personnel to take and store water samples throughout the hatchery season. It is difficult to know when a drop in larval production will occur which is why it’s essential to take these samples on a regular basis. The samples can later be sent to a commercial lab for specific testing that cannot be done on-site. The results from this testing could lead to solutions such as adaptive maneuvering or engineering strategies.

References

Goerlitz, D.F., E. Brown. (1972). “Methods for Analysis of Organic Substances in Water, Book 5, ChapterA3.” US Government Printing Office. Retrieved from https://pubs.usgs.gov/twri/05a03/report.pdf

Eaton, A. D., L.S. Clesceri, A.E. Greenberg, M.A.H. Franson, American Public Health Association., American Water Works Association., & Water Environment Federation. (1998). Standard methods for the examination of water and wastewater. Washington, DC: American Public Health Association.

United States Environmental Protection Agency. (2012 March). Monitoring and Assessment. Retrieved from US EPA website http://archive.epa.gov/water

Texas Commission on Environmental Quality. (2012 August). Collecting Water Samples. Retrieved from TCEQ website https://www.tceq.texas.gov/assests/pubic/commexec//pubs/rg/rg415/rg-415chapter5.pdf

Bricker, S. B., Rice, K.C., & Bricker III, O.P. (2014). From Headwaters to Coast: Influence of Human Activities on Water Quality of the Potomac River Estuary. Aquatic Geochemistry, 20(2), 291-323. doi: 10. 1007/s10498-014-9226-y

M. Vaudrey, T. Getches, K. Shaw, J. Markow, R. Britton. Effects of oyster depuration gear on eelgrass (Zostera marina L.) in a low density aquaculture site in long island sound. J. Shellfish Res., 28 (2009), pp. 243–250

Williamson, T. R., Tilley, D. R., & Campbell, E. (2015). Emergy analysis to evaluate the sustainability of two oyster aquaculture systems in the Chesapeake Bay. Ecological Engineering, 85, 103-120. doi:10.1016/j.ecoleng.2015.09.052

Ray, N. E., Li, J., Kangas, P. C., & Terlizzi, D. E. (2015). Water quality upstream and downstream of a commercial oyster aquaculture facility in Chesapeake Bay, USA. Aquacultural Engineering, 68, 35-42. doi:10.1016/j.aquaeng.2015.08.001

Rose, J. M., Bricker, S. B., Tedesco, M. A., & Wikfors, G. H. (2014). A role for shellfish aquaculture in coastal nitrogen management. Environmental Science & Technology, 48(5), 2519-2525. doi:10.1021/es4041336

Methratta, E. T., Menzie, C. A., Wickwire, W. T., & Richkus, W. A. (2013). Evaluating the risk of establishing a self-sustaining population of non-native oysters through large-scale aquaculture in Chesapeake Bay. Human and Ecological Risk Assessment: An International Journal, 19(5), 1234-1252. doi:10.1080/10807039.2013.767112

Top of Form

Appendices

a. Supporting Documents

| Table A. Commercial Laboratory Analytical Capabilities |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| List of commercial analytical labs1 |  | Analytical Capabilities2 |  |  |  |  |  |  |  |  |
|  |  | Matrix |  | Analyte |  |  |  |  |  |  |
| State | Lab Name | Solids | Water | Metals | Nutrients | Oil/Grease | Pesticides | Organics | Bacteria | Tributyltin |
| VA |  |  |  |  |  |  |  |  |  |  |
|  | A & L Easter Laboratories, Inc. | X |  | X | X |  |  |  |  |  |
|  | Air Water & Soil Laboratories, Inc. |  | X | X | X | X | X | X | X |  |
|  | Analytics Corporation | X | X | X | X | X | X | X |  |  |
|  | B&B Consultants, Inc. |  | X |  | X |  |  |  | X |  |
|  | Babcock & Wilcox Lynchburg Technology Center | X | X | X |  |  |  |  |  |  |
|  | E M S, Inc. |  | X |  | X |  |  |  | X |  |
|  | EnviroCompliance Laboratories, Inc.-Ashland | X | X | X | X | X |  | X | X |  |
|  | Environmental Hazards Service, LLC | X | X | X |  |  |  | X |  |  |
|  | Environmental Hazards Service, LLC | X | X | X |  |  |  | X |  |  |
|  | Environmental Monitoring Inc. |  | X | X | X | X |  |  | X |  |
|  | Environmental Systems Service, -Culpeper |  | X |  | X |  |  |  | X |  |
|  | Greenway Engineering, Inc | X | X |  | X |  |  | X |  |  |
|  | Hampton Roads Sanitation District Central | X | X | X | X | X | X | X | X |  |
|  | Inboden Environmental Services, Inc. |  | X |  | X | X |  |  | X |  |
|  | James R. Reed & Associates | X | X | X | X | X | X | X | X |  |
|  | Jennings Laboratories Inc. |  | X | X | X |  |  |  | X |  |
|  | Metropolitan Solutions | X | X | X |  |  |  |  |  |  |
|  | Newport News Shipbuilding-A Division of Huntington Ingalls Industries | X | X | X | X |  |  |  |  |  |
|  | Schneider Laboratories Global | X | X | X | X |  | X | X | X |  |
|  | Universal Laboratories- Hampton | X | X | X | X | X | X | X | X | X |
| MD |  |  |  |  |  |  |  |  |  |  |
|  | Enviro-Chem Laboratories, Inc | X | X | X |  |  |  |  |  |  |
|  | Martel Laboratories JDS |  | X | X | X | X | X | X |  |  |
|  | Maryland Spectral Services | X | X |  |  |  | X | X |  |  |
|  | Phase Separation Science | X | X | X | X | X | X | X |  |  |
|  | Water testing labs of Maryland-Salisbury |  | X |  |  | X |  |  | X |  |
| NC |  |  |  |  |  |  |  |  |  |  |
|  | Cape Fear Analytical, L.L.C. |  | X |  |  |  | X | X |  |  |
|  | Compuchem, A Division of Liberty Analytical | X | X | X | X | X | X | X |  |  |
|  | Element One, Inc | X | X | X |  |  |  |  |  |  |
|  | Environmental Conservation Laboratories-Cary | X | X | X | X |  | X | X |  |  |
|  | Pace Analytical Services-Asheville | X | X | X | X |  |  |  | X |  |
|  | Pace Analytical Services-Huntersville | X | X |  |  |  | X | X | X |  |
|  | Prism Laboratories | X | X | X | X | X | X | X |  |  |
|  | SGS North America | X | X | X |  |  | X | X |  |  |
| 1 Partial list compiled from VA Division of Consolidated Laboratory Services VELAP-certifies laboratories list (01-29-2015) |  |  |  |  |  |  |  |  |  |  |
| 2 Approximate; contact individual laboratories to confirm analytical capabilities for a particular sample type |  |  |  |  |  |  |  |  |  |  |