Design of Multimodal In-Vehicle Notifications at Highway-Rail Grade Crossings: A Perception Study

Objectives (491/500)

Accidents at Highway-Rail Grade Crossings (HRGCs) continue to be a worrisome issue, leading to great loss of life, productivity, medical, and legal costs that added up to nearly $40 million between 2008 and 2017 within the US alone (Hellman & Poirier, 2019). Within that same period, a total of 19,639 incidents were reported at different types of HRGCs, and 67% of them were due to motorists not stopping at an active HRGC (Hellman & Poirier, 2019). Driver behavior has long been cited as one of the main contributing factors to train-vehicle collisions (FRA, 2016). Only 20% of drivers have been reported to perform a double scan at active crossings, and 40% did so at passive crossings (Lautala et al., 2017). Risky behaviors such as trying to beat an approaching train despite being aware of the potential dangers, and familiarity with an HRGC causing drivers to be overconfident, are also common factors that lead to vehicle-train accidents (Yeh & Multer, 2008). Thus, the evidence suggests that there is a need to address driver distraction, and improve their behavior as they approach HRGCs.

To address this issue, and potentially reduce the likelihood of vehicle-train accidents at HRGCs, the current study investigates the design of novel visual and auditory in-vehicle alerts. Multimodal alerts can inform drivers to take appropriate action as they approach an HRGC. In the past, earcon and speech based in-vehicle auditory alerts (IVAA) have shown to improve driver behavior at HGRCs (Nadri et al., 2021, 2023). A prototype of a Rail Crossing Violation Warning (RCVW) system has also been developed that showed positive effects in reducing drivers’ HRGC approach speeds (Zhang et al., 2023). However, their combination still requires further research. In this work, a combination of visual and auditory alerts are developed, tested for their perceived effectiveness, and guidelines for how to best display them are suggested.

The design of five multimodal notifications were investigated in the current study. The visual content of the notifications were adapted from the RCVW system (Zhang et al., 2023) developed and tested previously, and the auditory content for each alert was adapted from past research on the IVAA system (Nadri et al., 2023) to improve driver behavior at both active and passive HRGCs. The five notifications were labelled Blue, Yellow_1, Yellow_2, Red_1, and Red_2. The purpose of the Blue notification was to simply inform drivers they were in vicinity of an HRGC. Yellow_1 asked drivers to slow down, and come to a stop at the HRGC. Yellow_2 informed drivers to complete turning at an intersection only after passing the HRGC. Red_1 was displayed to drivers who were travelling over the speed limit, and needed to quickly come to a stop at the HRGC. Red_2 was displayed when the driver was stopped over the HRGC tracks, and needed to clear the tracks immediately. A train was present for all the notifications; only active HRGCs with activated gates, lights, and bells were investigated in this study.

Approach (250/250)

Videos, showing a driver’s perspective as they approached an active HRGC in a rural environment, were recorded on a driving simulator. The visual content of the notifications were displayed on the instrument panel, presented on the right end of the screens. The Blue notification was investigated using a single factor design (Display Time – 20 vs. 40 sec to HRGC arrival). Notifications Yellow_1, Yellow_2, and Red_1 were investigated using a 2 (Display Time: 10 vs. 20 seconds to arrival at HRGC) x 2 (Speech Length – Long vs. Short) full factorial design. Notification Red_2 was investigated using a 2 (Speech Length – Long vs. Short) x 2 (Presentation Order – Sequential vs. Simultaneous) full factorial design. Sequential presentation displayed the earcon first, before speech; Simultaneous presentation displayed the earcon along with the duration of the speech. The audio for Blue notification did not contain any speech, but the audio for Yellow_1, Yellow_2, and Red_1 were always presented in a sequential manner.

Twenty-six participants who, on average, encountered a railroad crossing 5.6 times (SD = 10.6) a month were recruited for the study. Participants viewed the videos on the screens of a driving simulator. Participants were asked if they agreed or disagreed with statements regarding – Right Timing, Pleasantness, Urgency, Annoyance, Appropriateness, Unambiguity, Attention Capturing, Commanding, Startling, Willingness to Turn Off, and Distracting nature of the notifications on a scale of 1 (Strongly Disagree) – 5 (Strongly Agree). Hazard Level ratings were also collected using a scale of 1 (Notice), 2 (Caution), 3 (Warning), and 4 (Danger).

Findings (250/250)

For Blue, Display Time had a main effect. The 20 seconds display time  was rated higher for Right Timing, compared to 40 seconds. However, the 40 second display time was rated higher for Unambiguity.

For Yellow_1, Display Time and Speech Length had main effects. The 10 second notification was perceived as more Pleasing, less Distracting, and less likely to be Turned Off; short speech was rated as less annoying than long speech.

With Yellow_2, Display Time had a main effect; the 10 second notification was rated as more Pleasing. An interaction effect of Display Time and Speech Length showed that long speech at 10 seconds was less likely to be Turned Off, compared to short speech at 10 seconds. Similar effects were also observed for Distracting.

Regarding Red_1, Display Time had a main effect; the 10 second was rated as less Annoying and Distracting. An interaction effect of Display Time and Speech Length showed long speech at 20 seconds, as well as short speech at 10 seconds were rated higher compared to long speech at 10 seconds, and short speech at 20 seconds for Right Timing, Pleasantness, and Appropriateness, while the opposite effect was observed for likelihood to be Turned Off.

Finally, for Red_2, a main effect of Speech Length showed long speech was more likely to be Turned off. Further, a main effect of Presentation Order showed that simultaneous presentation was rated as more Urgent, Commanding, but also more Distracting, Annoying, and Startling.

All reported findings are statistically significant.

Takeaways (250/250)

The use of multimodal in-vehicle alerts is a novel methodology to improve driver behavior at HRGCs. Before implementation, or testing for their effectiveness, it is necessary to investigate the design of the components of the multimodal alerts systematically. The current work evaluated user perception of five multimodal alerts by varying their display time, speech length, and presentation order. Findings from the current study will help to further improve the visual and auditory components of multimodal alerts for use at HRGCs to potentially reduce vehicle-train accidents.

While it could be anticipated that a shorter length speech should be used in conjunction with a shorter display time, the results of this perception study show that this might be scenario dependent. For example, for the short storage HRGC scenario (Yellow_2 notification), participants preferred longer speech with a shorter arrival time. This could be because participants wanted to receive all possible information about the scenario from the notification, but also felt it was better suited for when they could compare the information with what they were able to directly observe at the HRGC. Similarly, Red_1 notification results suggest that depending on additional scenario variables, such as driver or train speed, visibility,  or advanced warning signs, both short and long speech might be appropriate with 10 and 20 second arrival times, respectively. Future research should involve testing the notifications by controlling for scenario related variables such as active and passive HGRCs, visibility conditions, and train presence, to test for their effectiveness in improving driver behavior.

References

FRA, O. of R. P. & D. (2016). ANALYSIS OF GRADE CROSSING ACCIDENTS RESULTING IN INJURIES AND FATALITIES (16–10; p. 4). Department of Transportation.

Hellman, A., & Poirier, P. (2019). Highway-Rail Intersection Crash Taxonomy for Connected Vehicle Safety Research—Ten Year Trend (Technical Report DOT-VNTSC-FRA-19-04; p. 45). U.S. Department of Transportation Office of the Assistant Secretary for Research and Technology John A. Volpe National Transportation Systems Center.

Lautala, P., Nelson, D., Dean, A., Jeon, M., & Muhire, M. (2017, September 17). Evaluating Driver Compliance at Highway Grade Crossings. Proceedings of the AREMA Annual Conference. AREMA Annual Conference, Indianapolis, IN, USA.

Nadri, C., Kekal, S., Li, Y., Li, X., Lee, S. C., Nelson, D., Lautala, P., & Jeon, M. (2023). “Slow down. Rail crossing ahead. Look left and right at the crossing”: In-vehicle auditory alerts improve driver behavior at rail crossings. Applied Ergonomics, 106, 103912. https://doi.org/10.1016/j.apergo.2022.103912

Nadri, C., Lee, S. C., Kekal, S., Li, Y., Li, X., Lautala, P., Nelson, D., & Jeon, M. (2021). Investigating the Effects of Earcon and Speech Variables on Hybrid Auditory Alerts at Rail Crossings. Proceedings of the 26th International Conference on Auditory Display (ICAD 2021), 38–43. https://doi.org/10.21785/icad2021.024

Yeh, M., & Multer, J. (2008). Driver Behavior at Highway-Railroad Grade Crossings: A Literature Review from 1990-2006 (Final Report DOT/FRA/ORD-08/03; p. 135). U.S. Department of Transportation Research and Innovative Technology Administration John A. Volpe National Transportation Systems Center. https://doi.org/10.1037/e621222011-001

Zhang, K., Lautala, P., Souleyrette, R. R., Tan, Y., Yang, Y., Hung, Y.-C., Mamun, T. I., Veinott, E. S., & Wang, T. (2023). Developing Safe and Efficient Driving and Routing Strategies at Railroad Grade Crossings based on Highway-Railway Connectivity (Technical Report DOT/FRA/ORD-23/14). Michigan Technological University, Houghton; The University of Kentucky.